NCTS-PDA4W Industrial Smart Handheld Terminal

Camera

NFC identification

QR code Reader

Waterproof, anti-fall, dustproof

- inch HD full screen · High-speed quad-core · Android 9.0 system

Bluetooth

WIFI

1D Code

Android

4G

i

GPS

Camera

NFC

PSAM

QR Code

WPC-9080 is developed based on Android 9.0 operating system, adopts high-performance processor, quad-core CPU, supports dual frequency WiFi and high-speed 4G communication, high running speed and saves power.

Users can develop customized functions according to your needs, helping users quickly realize information management and improve production efficiency.

Product parameters

Physical parameter

| Overall size | 157（H）*73（W）*21.6(T)mm（Scan head 26mm） |
| --- | --- |
| Total Weight | 280g（Including battery, weight varies in different configurations） |
| Display | 4 inches, 480 * 800 resolution, 16.7M colors |
| Touch screen | Industrial capacitive touch screen |
| Battery capacity | Polymer li-ion battery 4300mAh |
|  | Standby time> 300 hours |
|  | Working time> 12 hours（Depends on usage and network environment） |
|  | Charging time 3-4 hours (use the standard power adapter and data cable) |
| Expansion slot | 1 SIM card slot, 1 Micro SD (TF) card slot supports up to 128GB |
| Communication Interface | USB Type-C, Support OTG function, support Pump Express Plus fast charge |
| Audio | Microphone, handset, 1W speaker, support voice call |
| keyboard | 3 TP keys + 5 side keys + keyboard |
| Sensor | Light sensor, distance sensor, gravity sensor, vibration motor |
| Power Adapter | 100-240V，50-60Hz，0.29A；output：5V，2A |

Performance parameter

| High version | CPU: Cortex A-53 2.0 GHz Quad-core |
| --- | --- |
|  | RAM+ROM: 2GB+16GB |
| Extended memory | Micro SD (TF) card expandable to 128GB |

Development environment

| Operating system | Android P (Android 9.0) supports GMS certification |
| --- | --- |
| Development language | Java |
| Development tools | Eclipse/Android Studio |

Use environment

| Operating temperature | -20oC to 50oC |
| --- | --- |
| Storage temperature | -40oC to 70oC |
| Environment humidity | 5%RH - 95%RH（Non-condensing） |
| Drop specifications | 1.5m cement floor |
| Rolling test | 1000 rounds of 0.5 meter rolling (equivalent to 2000 impacts) |
| Electrostatic discharge (ESD) | ± 15kV air discharge, ± 8kV direct discharge |

| WLAN | Support IEEE802.11 ac / a / b / g / n protocol (2.4 / 5G dual frequency), |
| --- | --- |
| WWAN | GSM：850/900/1800/1900Mhz) |
|  | TD-SCDMA：Band34,Band39 |
|  | WCDMA：Band1,Band2,Band5,Band8 |
|  | TDD-LTE：Band38,Band39,Band40,Band41 |
|  | FDD-LTE：B1/B2/B3/B4/B5/B7/17/B20 |
| Bluetooth | Bluetooth 4.1（support HS） |
| GNSS | Integrated GPS, Beidou, GLONASS, built-in antenna, support AGPS |

Data collection

| 1D scanning engine | Honeywell N4313 / UE966 / Zebra SE965 |
| --- | --- |
| Support 1D barcode type | UPC/EAN,Code128,Code39,Code93,Code11, |
|  | Interleaved 2 of 5, Discrete 2 of 5, Chinese 2 of 5, |
|  | Codabar,MSI,RSS ect. |
| 2D scanning engine | Zebra SE4710 / Autonomous 2D engine |
| Support 2D barcode type | PDF417, MicroPDF417, Composite, RSS, TLC-39, |

NFC(Optional)

| Frequency | 13.56MHz |
| --- | --- |
| Protocol standard | ISO14443A/B, NFC-IP1, NFC-IP2 ect |
| Labeling standards | M1 card (S50, S70), CPU card, NFC tag, etc. |
| Reading distance | 2-4cm |

PSAM(Optional)

| PSAM | Support ISO7816 standard, 2 PSAM cards |
| --- | --- |